# CUSTOMER NAME :

# ORDER/QUOTE No. :

# Filter Vessel Safety Questionnaire

In order to meet new EC pressure vessel directives, we need to determine the level of inspection required for the proposed filter vessel. Therefore, we would be most grateful if you could complete the following questionnaire.

- Operating Temperature and Pressure

What is the Required design pressure? < 10 BAR

What is the Required design Temperature?  -1 to 9’c

# Fluid Danger Classification

## 1) It is necessary to determine whether the fluid to be filtered through the vessel is Group I or Group II:

What is the fluid to be filtered (chemical name)?  Fruit/Milk Based Drinks

Does the fluid fall into any of the following classifications? (Please Tick as Appropriate) NA

Explosive					Toxic	**

### Flammable category 1 or 2			Oxidising category 1, 2 or 3

Flammable category 3	*			Pyrophoric

*If Flammable category 3, go to section 1.1

** Acute oral toxicity category 1 and 2; acute dermal toxicity category 1 and 2; acute inhalation toxicity category 1,2 and 3; specific organ toxicity single exposure category 1.

  - If the fluid is flammable category 3, is the maximum allowable temperature above the flash point? ………..

If any of the above are ticked then the fluid is Group I, otherwise it is Group II (Except if Flammable category 3, then it is only Group I if the answer to 1.1  is YES)

Which group therefore is the fluid?  Group II

2) It is necessary to know whether the fluid is Liquid or Gaseous at the Design temperature stated in Section A i.e. if the vapour pressure is greater than 0.5 Bar above normal atmospheric pressure at the design temperature then it is considered a GAS, if not then it is a LIQUID.

Please tick as appropriate below.

The Fluid is Liquid ( the V.P. is <0.5 Bar) at the maximum design temperature	x

### The Fluid is Gaseous (the V.P. is >0.5 Bar) at the maximum design temperature

3) If the vessel will be under pressure when being purged with a non- dangerous gas such as air or nitrogen, then the vessel should be treated as being for use with group II gases. Final classification subject to answers to question I and II.

Is there a requirement for the vessel to be purged with non-dangerous gases such as air or nitrogen under pressure ? NO

If yes, what will be the pressure inside the vessel when purging? ………………. Bar

C)  ATEX

Whilst our vessels are outside the scope of the ATEX directive as they are classified as a ‘simple piece of equipment’, where our products are used in potentially explosive atmospheres some customers request our equipment to bear an ATEX mark (and therefore CE mark).

Does the vessel need to have an ATEX mark ? NO

Completed By Charlie Wilson-Vaughan		Date 14.1.26

Housing Serial No’s supplied against this questionnaire

…………………