| Customer & Plant Information |  |
| --- | --- |
| Contractor Information (Include contractor Name, Address, P#, Email): | Plant Information (Include Name, Address, P#, Email): |
| Contract Number: | Today’s date: |

Filter Installation Information

Are the operation and maintenance manuals on hand for your review? Yes No NA

Has Leopold’s packing lists been reviewed to ensure all necessary materials are onsite Yes No NA to complete the required installation work during visit?

Are the filter tanks clean? Yes No NA

“If required” have the filter tanks been hydrostatically tested Yes No NA

Are the filter tanks the correct dimensions? Yes No NA

Are there any obstructions in the filter tank or flume that could impede underdrain installation? Yes No NA

Has the backwash supply piping and flume been cleaned/flushed out? Yes No NA

Was the filter floor finished correctly to ensure good grout bonding? Yes No NA 1/8” or 3 to 4 millimeter (broom finish)

Have provisions been made to cover the filter during installation work when rain is in the forecast Yes No NA or when direct sunlight or high temperatures can cause the filter equipment to expand?

Have provisions been made to properly heat the installation area when temperatures go below Yes No NA 40 degrees to ensure proper curing of grout?

Are the following items on hand for installation work? (These are not supplied by Leopold) Yes No NA - Mortar Mixer (not a cement mixer) - Transit (should be a laser) - Portland Cement Type II (See Appendix1 in O&M) - Sand (See Appendix 1 in O&M ) - Sikaflex 1A or FDA and NF Approved equal - Pre-cut wedges ( may be 2” x 4” wood cut on-site ) - Epoxy for anchor rod installation: (If part of design) Suitable epoxy systems: “Sika”, “Hilti-Hit” or equal - Two rubber mallets - Two Torpedo levels - Two 4’ levels - Staple gun with 3/8” or ½” staples - Two flat shovels - Five 5-gallon buckets - Shop vac - Duct tape and roofing paper for covering laterals when grouting (With IMS cap) - ¼” Masonite to cover underdrain block when grouting (If IMS ® Cap is not part of design) - Chip or foam boards (to cover and protect laterals during and after installation) - Flashlight - Traffic cone - 5-7 workers

| Contractor Sign-Off |  |  |
| --- | --- | --- |
| Date: | Contractor representative name, title, and contact information: | Contractor representative signature: |